Fluid loss additives FC-631S

Chemical Safety Data Sheet

Section 1: Product Name

Product name: Fluid loss additive FC-631S.

Section 2: Hazard Overview

Physical/chemical hazard: non flammable and explosive products.

Health hazard: It has certain irritating effect on eyes and skin; Eating by mistake may cause irritation to the mouth and stomach.

Carcinogenicity: None.

Section 3: Composition/information on ingredients

| Type | Main component | Content | CAS NO. |
| --- | --- | --- | --- |
| FC-631S | Copolymer | 95-100% |  |
|  | Water | 0-5% | 7732-18-5 |

Section 4: First aid measures

Skin contact: Take off the contaminated clothes and wash with soapy water and flowing clean water.

Eye contact: Lift the eyelids and immediately wash them with a large amount of flowing water or normal saline. Seek medical attention in case of pain and itching.

Ingestion: Drink enough warm water to induce vomiting. Get medical attention if you feel unwell.

Inhalation: Leave the site to a place with fresh air. If breathing is difficult, seek medical advice.

Section 5: Firefighting measures

Combustion and explosion characteristics: Refer to section 9 "Physical and Chemical Properties".

Extinguishing agent: Foam, dry powder, carbon dioxide, water mist.

Section 6: Accidental release measures

Personal protective measures: Wear appropriate personal protective equipment. See Section 8 "Protective Measures".

Release: Try to collect the release and clean the leakage place.

Waste disposal: Bury properly or dispose according to local environmental protection requirements.

Packaging treatment: Transfer to the garbage station for proper treatment.

Section 7: Handling and Storage

Handling: Keep the container sealed and avoid skin and eye contact. Wear appropriate personal protective equipment.

Precautions for storage: It should be stored in a cool and dry place to prevent exposure to the sun and rain, and away from heat, fire and materials to be avoided.

Section 8: Exposure Control and Personal Protection

Engineering control: In most cases, good overall ventilation can achieve the purpose of protection.

Respiratory protection: Wear a dust mask.

Skin protection: Wear impermeable work clothes and protective gloves.

Eye/eyelid protection: Wear chemical safety goggles.

Other protection: Smoking, eating and drinking are prohibited at the work site.

Section 9: Physical and chemical properties

| Item | FC-631S |
| --- | --- |
| Color | White or light yellow |
| Characters | Powder |
| Odor | Non irritant |
| Water solubility | Water soluble |

Section 10: Stability and reactivity

Conditions to avoid: open fire, high heat.

Incompatible substance: oxidants.

Hazardous decomposition products: None.

Section 11: Toxicological Information

Invasion route: inhalation and ingestion.

Health hazard: ingestion may cause irritation to the mouth and stomach.

Skin contact: Long time contact may cause slight redness and itching of the skin.

Eye contact: cause eye irritation and pain.

Ingestion: cause nausea and vomiting.

Inhalation: cause cough and itching.

Carcinogenicity: None.

Section 12: Ecological Information

Degradability: The substance is not easily biodegradable.

Ecotoxicity: This product is slightly toxic to organisms.

Section 13: Disposal

Waste disposal method: bury properly or dispose according to local environmental protection requirements.

Contaminated packaging: it shall be handled by the unit designated by the environmental management department.

Section 14: Transport Information

This product is not listed in the International Regulations on the Transport of Dangerous Goods (IMDG, IATA, ADR/RID).

Packaging: The powder is packed in bags.

Section 15: Regulatory Information

Regulations on Safety Management of Hazardous Chemicals

Detailed Rules for the Implementation of the Regulations on the Safety Management of Hazardous Chemicals

Classification and Marking of Common Hazardous Chemicals (GB13690-2009)

General Rules for Storage of Common Hazardous Chemicals (GB15603-1995)

General technical requirements for transport packaging of Hazardous goods (GB12463-1990)

Section 16: Other Information

Issue date: 2020/11/01.

Revision date: 2020/11/01.

Recommended and restricted use: Please refer to other products and/or product application information. This product can only be used in industry.